|  | Напружено-деформований стан залізобетонних згинаних балок, зміцнених локальним пресуванням та підсилених зовнішнім обтиском |
| --- | --- |

Призначення та сфера застосування.  Будівництво

Основні характеристики, суть розробки. Проектне рішення направлено на підсилення існуючих несучих конструкцій та обґрунтування інноваційного відновлення їх несучої здатності. Запропоноване конструктивне рішення регульовано-напруженої балки, яка містить залізобетонне тіло і затяжку, закріплену по кінцях на балці, взаємодіючу посередині з натяжним елементом, що обпирається на нижню грань балки, і поперечну зовнішню арматуру, взаємодіючу у приопорних зонах балки з верхньою і нижньою її гранями, а в середній частині взаємодіючу з затяжкою. Поперечна арматура розтягнута, гнучка і розташована дзеркально симетрично в приопорних зонах балки з різними кутами нахилу до поздовжньої осі балки. Поперечна арматура балки, закріплена одним кінцем на верхній грані балки ближче до її середини з обмеженням зміщення до середини, а іншим кінцем на нижній грані балки ближче до її опор з обмеженням зміщення до опор і з можливістю поздовжнього розтягу верхньої грані балки і обтиску нижньої її грані та поперечного стиску приопорних зон. В 1/3 прольоту балки виконанане пресування бетону в середній частині з різною інтенсивністю ля порівняння ефективності.

Запропонована саморегульована, проста і надійна нова конструкція балок, що ефективно підсилюються при дії на неї зовнішнього навантаження шляхом раціонального перерозподілу напружень між стисненою та розтягнутою зонами за допомогою легких і гнучких елементів, що працюють на розтяг та додаткового зміцнення середньої частини підсилюваного елемента.

Порівняння зі світовими аналогами, основні переваги розробки. Зовнішнє попереднє напруження у вигляді затяжок на сучасному етапі стало одним з найпривабливіших методів для зміцнення стальних, бетонних та сталебетонних конструкцій.  Воно дозволяє проектувальникам створювати конструкції, які мають кращу міцність і довговічність за рахунок підсилення у разі погіршення їх стану чи необхідності збільшення або відновлення їх несучої здатності. На практиці зовнішнє попереднє напруження у вигляді затяжок ефективно застосовується при улаштуванні балкових конструкцій покриттів і перекриттів будівель та споруд, прогонів мостів, а також при їх реконструкції чи підсиленні, обсяги яких в останні роки у зв’язку зі збільшенням їх фізичного зносу значно зросли. Так, проектування чи підсилення балок з використанням затяжок дає переваги, які дозволяють: зменшити висоту перерізів елементів; проводити улаштування підсилення під дією зовнішнього навантаження; здійснювати контроль та регулювання попереднього напруження; скоротити термін проведення будівельних і ремонтних робіт за рахунок простоти установки і улаштування затяжок без перерви технологічного процесу. В той же час залізобетонні згинані елементи з попередньо напруженими затяжками на сьогодні мало застосовуються у зв’язку з відсутністю узагальнюючої методики їх розрахунку.  За останні роки методи розрахунку залізобетонних елементів дістали подальшого розвитку на основі деформаційної моделі, яка враховує реальні діаграми роботи бетону й арматури.

Стан охорони інтелектуальної власності. Пат.99090 Україна, МПК E04C 3/20, E04G 23/02. Регульовано-напружена балка Чекановича / М.Г. Чеканович. - № a201414006, заявл. 26.12.2014; опубл. 25.05.2015, Бюл.№ 10, 2015. Комплекс  «Кращий винахід України 2016 року» диплом від 21 квітня 2017 року.

Затребуваність на ринку. Балкові елементи в будівництві складають значну частину будівельних конструкцій. Серед них ригелі, плити покриття і перекриття, підкранові балки цехів, фундаментні балки, прогони мостів, шляхопроводів та багато інших. Підвищення міцності таких конструкцій може скоротити матеріальні витрати і підвищити ефективність будівництва. Розробка дозволяє до 30-40% зменшити матеріальні витрати у будівельному комплексі України на балкові конструкції.

Стан готовності роботи. Виконані дослідження залізобетонних згинаних балок зміцнених локальним пресуванням та підсилених зовнішнім обтиском в лабораторії будівельних матеріалів та конструкцій кафедри будівництва ДВНЗ «ХДАУ».

Координати для зв’язку: 73006, м. Херсон, ДВНЗ «Херсонський державний аграрний університет», (0552) 41-75-92, e-mail: simonova_ok@ukr.net

Stress-strain state of reinforced concrete bendable beams mixed by local management and reinforced by external crimping

Purpose and scope of application. Construction

Main features, the essence of the development. The design solution is aimed at strengthening the existing load-bearing structures and substantiating the innovative restoration of their load-bearing capacity. The proposed constructive solution is adjustable-intense beams, which contains a reinforced concrete body and tight, fastened at the ends to a beam in the middle usabout with tensioning element, which rests on the bottom face of the beam, and transverse external reinforcement, usaimdur in primarnih zones of a beam with upper and lower faces, and in the middle part interacts with the puff. The cross armature is stretched, flexible and positioned mirror-symmetrically in the support zones of the beam with different angles of inclination to the longitudinal axis of the beam. Shear reinforcement of a beam fixed at one end on the top edge of the beam closer to the middle with the offset constraint to the middle, and the other end on the bottom face of the beam near the supports, with the restriction of displacement of supports and with the possibility of longitudinal stretching of the upper face of the beam and crimping its lower edge and transverse compression primarnih zones. In 1/3 of the span of the beam, concrete was pressed in the middle part with different intensities to compare the efficiency.

A self-regulating, simple and reliable new design of beams is proposed, which is effectively strengthened by the external load on it by rationally redistributing the stresses between the compressed and stretched zones with the help of light and flexible elements working on tension and additional strengthening of the middle part of the reinforced element.

Comparison with world analogues, the main advantages of the development.

External pre-stress in the form of puffs at the present stage has become one of the most attractive methods for strengthening steel, concrete and steel-concrete structures. It allows designers to create structures that have better strength and durability due to reinforcement in the event of deterioration of their condition or the need to increase or restore their load-bearing capacity. In practice, external pre-stress in the form of puffs is effectively used in the construction of beam structures of coverings and floors of buildings and structures, bridge spans, as well as in their reconstruction or strengthening, the volume of which has increased significantly in recent years due to the increase in their physical wear. So, designing or strengthening beams using puffs gives benefits that allow to reduce the height of the element sections; carry out amplification device under the action of external loads; control and regulation of pre-stress; to reduce the period of carrying out construction and repair works due to the ease of installation and device puffs without interruption of the process. At the same time, reinforced concrete bending elements with prestressed puffs aren`t widely used today due to the lack of a generalizing method for their calculation. In recent years, methods for calculating reinforced concrete elements have been further developed on the basis of a deformation model that takes into account real diagrams of concrete and rebar operation.

State of intellectual property protection. Pat. 99090 Ukraine, IPC E04C 3/20, E04G 23/02. Regulated-strained chekanovich beam / M. G. Chekanovich. - no. a201414006, application 26.12.2014; publ. 25.05.2015, bul. no. 10, 2015. Complex "Best invention of Ukraine 2016" diploma from April 21, 2017.

Demand in the market. Beam elements in construction make up a significant part of building structures. Among them are crossbars, slabs and floor slabs, crane beams of workshops, Foundation beams, girders of bridges, overpasses and many others. Increasing the strength of such structures can reduce material costs and improve construction efficiency. The development allows you to reduce material costs in the construction complex of Ukraine for beam structures by up to 30-40%.

The state of readiness of the work. Performed research of reinforced concrete beams reinforced by local pressing and reinforced by external crimping in the laboratory of building materials and structures of the Department of construction of the state technical University "KSAU".

Contact coordinates: 73006, t. Kherson, Kherson State Agrarian University , (0552) 41-75-92, e-mail: simonova_ok@ukr.net